Aluminium Screw and Nut

A beginner’s lathe project

Copyright: Sutton Grammar School 01/11/2017

This project is intended for use by those who have been shown the basics of how a lathe works and have received appropriate Health and Safety training on its use. It is not designed to produce a usable item, more an ornament. The oversize head and nut are to aid production using standard 18mm AF Aluminium hex rod that is available.

Materials needed:

100mm length of 18mm AF hex rod should be the maximum needed per pupil.

55mm M10 Hex Head Partially Threaded Screw with Oversize Head

Tooling needed:

Right Handed turning tool;

Parting tool;

M10x1.5mm Die and holder;

AnchorLube;

File;

Deburring tool.

Production process:

- Insert 18mmAF Hex Rod into 3-Jaw chuck and secure;

- Using Right Handed Turning Tool, face the end of the Hex Rod (c.535rpm);

- Mark the Hex Rod 55mm from the end;

- Using Right Handed Turning Tool, turn down the first 55mm to a diameter of 10mm (c.1350rpm);

- Using the M10 Die and holder, thread the first 25mm of the screw blank;

- Mark the Head of the Screw on the remaining Hex Rod at a distance of 7mm;

- Using the Parting Tool to the left of the marked line to part off the Screw (use a low speed c. (use a low speed c. 125rpm) and use Anchorlube diluted 3:1 with water;

- With a file, deburr the Head of the Screw.

M10 Nut with Oversize Head

Tooling needed:

Right Handed turning tool;

Parting tool;

Centre Drill and Drill Chuck,

Taper Shank 8.5mm Drill Bit;

M10x1.5mm Tap and holder;

AnchorLube;

File;

Deburring tool;

Countersink.

Production process:

- Insert 18mmAF Hex Rod into 3-Jaw Chuck and secure;

- Using Right Handed Turning Tool, face the end of the rod;

- Adjust the Compound Slide to use the Right Handed Turning Tool to chamfer the end of Hex Rod;

- Reset the Compound Slide to 90o

- Mark the Hex Rod at a distance of 10mm from the end;

- Using the Parting Tool to the left of the marked line to part off the nut blank (use a low speed c. 125rpm and Anchorlube diluted 3:1 with water);

- With a file, deburr the nut blank.

- Insert the nut blank into 3-Jaw Chuck and secure;

- Adjust the Compound slide to use the Right Handed Turning Tool to chamfer the end flat side of the nut blank;

- Using the Centre Drill in the Tailstock mounted Drill Chuck, drill to a depth of 3mm;

- Remove the Drill Chuck from the Tailstock;

- Insert Taper Shank 8.5mm Drill Bit into the Tailstock;

- Using the 8.5mm Drill, drill through the nut blank;

- Remove the nut from the 3 Jaw Chuck;

- Using the countersink on the holes on both sides of the nut;

- Insert the nut blank back into 3-Jaw chuck and secure;

- Apply some neat AnchorLube to the M10 Tap.

- Using the M10 Tap and holder, thread the nut fully;

- Remove completed nut.